# Visual object recognition

You can run this tutorial on:

- ROSbot 2.0

- ROSbot 2.0 PRO

- ROSbot 2.0 simulation model (Gazebo)

## Introduction

Objects can be recognized by a robot with use of a vision system. It is based on image characteristics like points, lines, edges colours and their relative positions.

Processing of object recognition consists of two steps. First is teaching and should be executed before main robot operation. During this step object is presented to the vision system, image and extracted set of features are saved as a pattern. Many objects can be presented to the system.

Second step is actual recognition which is executed constantly during robot operation. Every frame of camera is processed, image features are extracted and compared to data set in the memory. If enough features matches the pattern, then the object is recognized.

In this tutorial we will use find_object_2d node from find_object_2d package for both teaching and recognition.

As an image source we will use nodes from astra.launch as in tutorial 1.

## Teaching objects

Anything could be an object to recognize, but remember, that the more edges and contrast colours it has, the easier it will be recognized. A piece of paper with something drawn on it would be enough for this tutorial.

First you should run find_object_2d and astra.launch. Node find_object_2d by default subscribe to image topic, you should remap it to topic /camera/rgb/image_raw.

You can use below launch file:

<launch>

<arg name="use_rosbot" default="false"/>

<arg name="use_gazebo" default="true"/>

<arg name="teach" default="true"/>

<arg name="recognize" default="false"/>

<arg if="$(arg teach)" name="chosen_world" value="rosbot_world_teaching"/>

<arg if="$(arg recognize)" name="chosen_world" value="rosbot_world_recognition"/>

<include if="$(arg use_rosbot)" file="$(find astra_launch)/launch/astra.launch"/>

<include if="$(arg use_gazebo)" file="$(find rosbot_gazebo)/launch/$(arg chosen_world).launch"/>

<include if="$(arg use_gazebo)" file="$(find rosbot_gazebo)/launch/rosbot.launch"/>

<node name="teleop_twist_keyboard" pkg="teleop_twist_keyboard" type="teleop_twist_keyboard.py" output="screen"/>

<node pkg="find_object_2d" type="find_object_2d" name="find_object_2d">

<remap from="image" to="/camera/rgb/image_raw"/>

<param name="gui" value="$(arg teach)"/>

<param if="$(arg recognize)" name="objects_path" value="$(find tutorial_pkg)/image_rec/"/>

</node>

</launch>

Copy

To start teaching objects on ROSbot:

roslaunch tutorial_pkg tutorial_4.launch use_rosbot:=true use_gazebo:=false teach:=true recognize:=false

Copy

To start teaching objects in Gazebo:

roslaunch tutorial_pkg tutorial_4.launch use_rosbot:=false use_gazebo:=true teach:=true recognize:=false

Copy

To place objects in front of camera using Gazebo, you can use buttons Translation mode and Rotation modein top left corner and drag objects to desired position. Another option is to drive ROSbot to look at the selected object.

After launching the find_object_2d node with properly adjusted image topic, new window should appear:

On the left side of the window there are thumbnails of saved images (should be empty at first run). Application main window contains camera view. Yellow circles on the image are marking extracted image features.

To begin teaching process choose from the main toolbar Edit → Add object from scene.... New window will appear:

Now move the object and camera in order to cover as many features of the object as possible. While doing that try not to catch object surroundings. When it’s done, click Take picture.

In next view click Select region and select only that part of taken picture, that covers desired object and click Next.

You will get confirmation of features extracted from selected image area. If presented image is in accordance with what you selected, click End

You should see new thumbnail in the left panel. Notice the number outside of parentheses on the left of the image, this is the object ID.

Now you can add some more objects to be recognized. Remember their IDs, you will need them later:

When you have enough objects in the database choose from the main toolbar File → Save objects... and choose a folder to store recognized objects. Close the window and stop all running nodes.

## Recognizing objects

Objects will be recognized by the same node which was used for teaching but it works in slightly different configuration. We will set two new parameters for the node. For parameter gui we will set value false, this will run node without window for learning objects as we no longer need it. Another parameter will be objects_path, this should be a path to a folder that you have just chosen to store recognized objects.

You can use the same launch file as for teaching, but with different parameter values.

On ROSbot:

roslaunch tutorial_pkg tutorial_4.launch use_rosbot:=true use_gazebo:=false teach:=false recognize:=true

Copy

In Gazebo:

roslaunch tutorial_pkg tutorial_4.launch use_rosbot:=false use_gazebo:=true teach:=false recognize:=true

Copy

Node is publishing to /objects topic with message type std_msgs/Float32MultiArray. Data in this message is covering: object ID, size of recognized object and its orientation. The most interesting for us is object ID, this is first element of array.

Whenever object is recognized, it will be published in that topic. Place different objects in front of camera and observe as their data is published in the topic.

To watch messages published in the topic, you can use rostopic tool:

rostopic echo /objects

Copy

## Making decision with recognized object

To perform a robot action based on recognized object, we will make a new node. It’s task will be to subscribe /objects topic and publish message to /cmd_vel with speed and direction depending on the object.

Create a new file, name it action_controller.cpp and place it in src folder under tutorial_pkg. Then open it in text editor and paste below code:

#include <ros/ros.h>

#include <std_msgs/Float32MultiArray.h>

#include <geometry_msgs/Twist.h>

#define SMILE 4

#define ARROW_LEFT 3

#define ARROW_UP 5

#define ARROW_DOWN 6

int id = 0;

ros::Publisher action_pub;

geometry_msgs::Twist set_vel;

void objectCallback(const std_msgs::Float32MultiArrayPtr &object)

{

if (object->data.size() > 0)

{

id = object->data[0];

switch (id)

{

case ARROW_LEFT:

set_vel.linear.x = 0;

set_vel.angular.z = 1;

break;

case ARROW_UP:

set_vel.linear.x = 1;

set_vel.angular.z = 0;

break;

case ARROW_DOWN:

set_vel.linear.x = -1;

set_vel.angular.z = 0;

break;

default: // other object

set_vel.linear.x = 0;

set_vel.angular.z = 0;

}

action_pub.publish(set_vel);

}

else

{

// No object detected

set_vel.linear.x = 0;

set_vel.angular.z = 0;

action_pub.publish(set_vel);

}

}

int main(int argc, char **argv)

{

ros::init(argc, argv, "action_controller");

ros::NodeHandle n("~");

ros::Rate loop_rate(50);

ros::Subscriber sub = n.subscribe("/objects", 1, objectCallback);

action_pub = n.advertise<geometry_msgs::Twist>("/cmd_vel", 1);

set_vel.linear.x = 0;

set_vel.linear.y = 0;

set_vel.linear.z = 0;

set_vel.angular.x = 0;

set_vel.angular.y = 0;

set_vel.angular.z = 0;

while (ros::ok())

{

ros::spinOnce();

loop_rate.sleep();

}

}

Copy

Below is an explanation of the code line by line.

Including required headers:

#include <ros/ros.h>

#include <std_msgs/Float32MultiArray.h>

#include <geometry_msgs/Twist.h>

Copy

Defining constants for recognized objects, adjusting values to IDs of objects recognized by your system:

#define SMILE 8

#define ARROW_LEFT 9

#define ARROW_UP 10

#define ARROW_DOWN 11

Copy

Integer for storing object identifier:

int id = 0;

Copy

Publisher for velocity commands:

ros::Publisher action_pub;

Copy

Velocity command message:

geometry_msgs::Twist set_vel;

Copy

Callback function for handling incoming messages with recognized objects data:

void objectCallback(const std_msgs::Float32MultiArrayPtr &object) {

Copy

Checking if size of data field is non zero, if it is, then object is recognized. When data field size is zero, then no object was recognized.

if (object->data.size() > 0) {

Copy

Reading id of recognized object:

id = object->data[0];

Copy

Depending on recognized object, setting appropriate speed values:

switch (id)

{

case ARROW_LEFT:

set_vel.linear.x = 0;

set_vel.angular.z = 1;

break;

case ARROW_UP:

set_vel.linear.x = 1;

set_vel.angular.z = 0;

break;

case ARROW_DOWN:

set_vel.linear.x = -1;

set_vel.angular.z = 0;

break;

default: // other object

set_vel.linear.x = 0;

set_vel.angular.z = 0;

}

Copy

Publishing velocity command message:

action_pub.publish(set_vel);

Copy

Stopping all motors when no object was detected:

else

{

// No object detected

set_vel.linear.x = 0;

set_vel.angular.z = 0;

action_pub.publish(set_vel);

}

Copy

Main function, node initialization and setting main loop interval:

int main(int argc, char **argv)

{

ros::init(argc, argv, "action_controller");

ros::NodeHandle n("~");

ros::Rate loop_rate(50);

Copy

Subscribing to /objects topic:

ros::Subscriber sub = n.subscribe("/objects", 1, objectCallback);

Copy

Preparing publisher for velocity commands:

action_pub = n.advertise<geometry_msgs::Twist>("/cmd_vel", 1);

Copy

Setting zeros for initial speed values:

set_vel.linear.x = 0;

set_vel.linear.y = 0;

set_vel.linear.z = 0;

set_vel.angular.x = 0;

set_vel.angular.y = 0;

set_vel.angular.z = 0;

Copy

Main loop, waiting for trigger messages:

while (ros::ok())

{

ros::spinOnce();

loop_rate.sleep();

}

Copy

Last thing to do is editting the CMakeLists.txt file. Find line:

add_executable(tutorial_pkg_node src/tutorial_pkg_node.cpp)

Copy

and add below code after it:

add_executable(action_controller_node src/action_controller.cpp)

Copy

Find also:

target_link_libraries(tutorial_pkg_node

${catkin_LIBRARIES}

)

Copy

and add below code after it:

target_link_libraries(action_controller_node

${catkin_LIBRARIES}

)

Copy

Now you can build your node and test it.

Task 1 Run your node along with find_object_2d and astra.launch or Gazebo simulator. Then use rosnode, rostopic and rqt_graph tools to examine the system. Place different objects in front of your robot one by one. Observe how it drives and turns depending on differnt objects.

You can use below launch file:

<launch>

<arg name="use_rosbot" default="true"/>

<arg name="use_gazebo" default="false"/>

<arg name="teach" default="false"/>

<arg name="recognize" default="true"/>

<arg if="$(arg teach)" name="chosen_world" value="rosbot_world_teaching"/>

<arg if="$(arg recognize)" name="chosen_world" value="rosbot_world_recognition"/>

<include if="$(arg use_rosbot)" file="$(find astra_launch)/launch/astra.launch"/>

<include if="$(arg use_gazebo)" file="$(find rosbot_gazebo)/launch/$(arg chosen_world).launch"/>

<include if="$(arg use_gazebo)" file="$(find rosbot_gazebo)/launch/rosbot.launch"/>

<node name="teleop_twist_keyboard" pkg="teleop_twist_keyboard" type="teleop_twist_keyboard.py" output="screen"/>

<node pkg="find_object_2d" type="find_object_2d" name="find_object_2d">

<remap from="image" to="/camera/rgb/image_raw"/>

<param name="gui" value="$(arg teach)"/>

<param if="$(arg recognize)" name="objects_path" value="$(find tutorial_pkg)/image_rec/"/>

</node>

<node pkg="tutorial_pkg" type="action_controller_node" name="action_controller" output="screen"/>

</launch>

Copy

## Getting position of recognized object

Besides the ID of recognized object, find_object_2d node is also publishing a homography matrix of recognized object. In computer vision homography is used to define position of object relative to the camera. We will use it to obtain horizontal position of the object. Homography matrix is published in the same topic as the ID, but in the next cells of array, they are formatted as

[objectId1, objectWidth, objectHeight, h11, h12, h13, h21, h22, h23, h31, h32, h33, object2].

We will modify node to rotate robot to the direction of recognized object.

Open action_controller.cpp file in text editor.

Begin with including of required header file:

#include <opencv2/opencv.hpp>

Copy

Variable for storing camera centre- this should be half of your camera horizontal resolution:

int camera_center = 320; // left 0, right 640

Copy

Variables for defining rotation speed:

float max_ang_vel = 0.6;

float min_ang_vel = 0.4;

float ang_vel = 0;

Copy

Variable for object width and height:

float objectWidth = object->data[1];

float objectHeight = object->data[2];

Copy

Variable for storing calculated object centre:

float x_pos;

Copy

Variable defining how much rotation speed should increase with every pixel of object displacement:

float speed_coefficient = (float) camera_center / max_ang_vel /4;

Copy

Object for homography matrix:

cv::Mat cvHomography(3, 3, CV_32F);

Copy

Vectors for storing input and output planes:

std::vector<cv::Point2f> inPts, outPts;

Copy

Adding new case in switch statement:

case SMILE:

Copy

Extracting coefficients homography matrix:

cvHomography.at<float>(0, 0) = object->data[3];

cvHomography.at<float>(1, 0) = object->data[4];

cvHomography.at<float>(2, 0) = object->data[5];

cvHomography.at<float>(0, 1) = object->data[6];

cvHomography.at<float>(1, 1) = object->data[7];

cvHomography.at<float>(2, 1) = object->data[8];

cvHomography.at<float>(0, 2) = object->data[9];

cvHomography.at<float>(1, 2) = object->data[10];

cvHomography.at<float>(2, 2) = object->data[11];

Copy

Defining corners of input plane:

inPts.push_back(cv::Point2f(0, 0));

inPts.push_back(cv::Point2f(objectWidth, 0));

inPts.push_back(cv::Point2f(0, objectHeight));

inPts.push_back(cv::Point2f(objectWidth, objectHeight));

Copy

Calculating perspective transformation:

cv::perspectiveTransform(inPts, outPts, cvHomography);

Copy

Calculating centre of object from its corners:

x_pos = (int) (outPts.at(0).x + outPts.at(1).x + outPts.at(2).x + outPts.at(3).x) / 4;

Copy

Calculating angular speed value proportional to position of object and putting it into velocity message:

ang_vel = -(x_pos - camera_center) / speed_coefficient;

if (ang_vel >= -(min_ang_vel / 2) && ang_vel <= (min_ang_vel / 2))

{

set_vel.angular.z = 0;

}

else if (ang_vel >= max_ang_vel)

{

set_vel.angular.z = max_ang_vel;

}

else if (ang_vel <= -max_ang_vel)

{

set_vel.angular.z = -max_ang_vel;

}

else

{

set_vel.angular.z = ang_vel;

}

Copy

Your final file should look like this:

#include <ros/ros.h>

#include <std_msgs/Float32MultiArray.h>

#include <geometry_msgs/Twist.h>

#include <std_msgs/String.h>

#include <opencv2/opencv.hpp>

#define SMILE 4

#define ARROW_LEFT 3

#define ARROW_UP 5

#define ARROW_DOWN 6

int id = 0;

ros::Publisher action_pub;

geometry_msgs::Twist set_vel;

int camera_center = 320; // left 0, right 640

float max_ang_vel = 0.6;

float min_ang_vel = 0.4;

float ang_vel = 0;

void objectCallback(const std_msgs::Float32MultiArrayPtr &object)

{

if (object->data.size() > 0)

{

id = object->data[0];

float objectWidth = object->data[1];

float objectHeight = object->data[2];

float x_pos;

float speed_coefficient = (float)camera_center / max_ang_vel / 4;

// Find corners OpenCV

cv::Mat cvHomography(3, 3, CV_32F);

std::vector<cv::Point2f> inPts, outPts;

switch (id)

{

case ARROW_LEFT:

set_vel.linear.x = 0;

set_vel.angular.z = 1.0;

break;

case ARROW_UP:

set_vel.linear.x = 1;

set_vel.angular.z = 0;

break;

case ARROW_DOWN:

set_vel.linear.x = -1;

set_vel.angular.z = 0;

break;

case SMILE:

cvHomography.at<float>(0, 0) = object->data[3];

cvHomography.at<float>(1, 0) = object->data[4];

cvHomography.at<float>(2, 0) = object->data[5];

cvHomography.at<float>(0, 1) = object->data[6];

cvHomography.at<float>(1, 1) = object->data[7];

cvHomography.at<float>(2, 1) = object->data[8];

cvHomography.at<float>(0, 2) = object->data[9];

cvHomography.at<float>(1, 2) = object->data[10];

cvHomography.at<float>(2, 2) = object->data[11];

inPts.push_back(cv::Point2f(0, 0));

inPts.push_back(cv::Point2f(objectWidth, 0));

inPts.push_back(cv::Point2f(0, objectHeight));

inPts.push_back(cv::Point2f(objectWidth, objectHeight));

cv::perspectiveTransform(inPts, outPts, cvHomography);

x_pos = (int)(outPts.at(0).x + outPts.at(1).x + outPts.at(2).x +

outPts.at(3).x) /

4;

ang_vel = -(x_pos - camera_center) / speed_coefficient;

if (ang_vel >= -(min_ang_vel / 2) && ang_vel <= (min_ang_vel / 2))

{

set_vel.angular.z = 0;

}

else if (ang_vel >= max_ang_vel)

{

set_vel.angular.z = max_ang_vel;

}

else if (ang_vel <= -max_ang_vel)

{

set_vel.angular.z = -max_ang_vel;

}

else

{

set_vel.angular.z = ang_vel;

}

break;

default: // other object

set_vel.linear.x = 0;

set_vel.angular.z = 0;

}

action_pub.publish(set_vel);

}

else

{

// No object detected

set_vel.linear.x = 0;

set_vel.angular.z = 0;

action_pub.publish(set_vel);

}

}

int main(int argc, char **argv)

{

std_msgs::String s;

std::string str;

str.clear();

str.append("");

std::to_string(3);

s.data = str;

ros::init(argc, argv, "action_controller");

ros::NodeHandle n("~");

ros::Subscriber sub = n.subscribe("/objects", 1, objectCallback);

ros::Rate loop_rate(50);

action_pub = n.advertise<geometry_msgs::Twist>("/cmd_vel", 1);

set_vel.linear.x = 0;

set_vel.linear.y = 0;

set_vel.linear.z = 0;

set_vel.angular.x = 0;

set_vel.angular.y = 0;

set_vel.angular.z = 0;

while (ros::ok())

{

ros::spinOnce();

loop_rate.sleep();

}

}

Copy

Last thing to do is to edit the CMakeLists.txt file. Find line:

find_package(catkin REQUIRED COMPONENTS roscpp )

Copy

and add below code after it:

find_package( OpenCV REQUIRED )

Copy

Find also:

include_directories(

${catkin_INCLUDE_DIRS}

)

Copy

and change it to:

include_directories(

${catkin_INCLUDE_DIRS}

${OpenCV_INCLUDE_DIRS}

)

Copy

Find:

target_link_libraries(action_controller_node

${catkin_LIBRARIES}

)

Copy

and change it to:

target_link_libraries(action_controller_node

${catkin_LIBRARIES}

${OpenCV_LIBRARIES}

)

Copy

Now you can build your node and test it.

Task 2 Run your node along with find_object_2d and astra.launch or Gazebo simulator. Place the object with ID bonded to new case in switch statement in front of your robot. Observe how it turns towards the object.

## Following the object

In this section you will modify your robot to turn and also drive towards the object while keeping distance to it. For keeping the distance we will use one of two different types of sensors.

Log in to Husarion Cloud and open project that you created in previous manual. You will need to edit it a little.

Include required header files:

void initDistanceSensorsPublisher()

{

range_fl.header.frame_id = "range_fl";

range_fr.header.frame_id = "range_fr";

range_rl.header.frame_id = "range_rl";

range_rr.header.frame_id = "range_rr";

switch (sensor_type)

{

case SENSOR_LASER:

range_fl.field_of_view = 0.26;

range_fl.min_range = 0.03;

range_fl.max_range = 0.90;

range_fr.field_of_view = 0.26;

range_fr.min_range = 0.03;

range_fr.max_range = 0.90;

range_rl.field_of_view = 0.26;

range_rl.min_range = 0.03;

range_rl.max_range = 0.90;

range_rr.field_of_view = 0.26;

range_rr.min_range = 0.03;

range_rr.max_range = 0.90;

break;

case SENSOR_INFRARED:

range_fl.radiation_type = sensor_msgs::Range::INFRARED;

range_fl.field_of_view = 0.26;

range_fl.min_range = 0.05;

range_fl.max_range = 0.299;

range_fr.radiation_type = sensor_msgs::Range::INFRARED;

range_fr.field_of_view = 0.26;

range_fr.min_range = 0.05;

range_fr.max_range = 0.299;

range_rl.radiation_type = sensor_msgs::Range::INFRARED;

range_rl.field_of_view = 0.26;

range_rl.min_range = 0.05;

range_rl.max_range = 0.299;

range_rr.radiation_type = sensor_msgs::Range::INFRARED;

range_rr.field_of_view = 0.26;

range_rr.min_range = 0.05;

range_rr.max_range = 0.299;

break;

case NO_DISTANCE_SENSOR:

// Do your own implementation

break;

}

if (sensor_type != SensorType::NO_DISTANCE_SENSOR)

{

range_pub_fl = new ros::Publisher("/range/fl", &range_fl);

range_pub_fr = new ros::Publisher("/range/fr", &range_fr);

range_pub_rl = new ros::Publisher("/range/rl", &range_rl);

range_pub_rr = new ros::Publisher("/range/rr", &range_rr);

nh.advertise(*range_pub_fl);

nh.advertise(*range_pub_fr);

nh.advertise(*range_pub_rl);

nh.advertise(*range_pub_rr);

}

}

Copy

Assign values to variables, put into messages and publish them:

if (sensor_type != SensorType::NO_DISTANCE_SENSOR)

{

// get ranges from distance sensors

ranges = rosbot.getRanges(sensor_type);

range_fl.range = ranges[0];

range_fr.range = ranges[1];

range_rl.range = ranges[2];

range_rr.range = ranges[3];

Serial.printf("Ranges %f %f %f %f\n", ranges[0], ranges[1], ranges[2], ranges[3]);

// publish ranges

range_pub_fl->publish(&range_fl);

range_pub_fr->publish(&range_fr);

range_pub_rl->publish(&range_rl);

range_pub_rr->publish(&range_rr);

}

Copy

Whole file should look like this:

#include "hFramework.h"

#include "hCloudClient.h"

#include "ros.h"

#include "geometry_msgs/Twist.h"

#include "geometry_msgs/PoseStamped.h"

#include "sensor_msgs/BatteryState.h"

#include "std_msgs/Bool.h"

#include "sensor_msgs/Range.h"

#include "tf/tf.h"

#include "ROSbot.h"

using namespace hFramework;

// Uncomment one of these lines, accordingly to range sensor type of your ROSbot

// If you have version with infrared sensor:

// static const SensorType sensor_type = SENSOR_INFRARED;

// If you have version with laser sensor:

static const SensorType sensor_type = SENSOR_LASER;

// If you want to use your own sensor:

// static const SensorType sensor_type = NO_DISTANCE_SENSOR;

// Uncomment one of these lines, accordingly to IMU sensor type of your device

// If you have version with MPU9250:

static const ImuType imu_type = MPU9250;

// If you want to use your own sensor:

// static const ImuType imu_type = NO_IMU;

ros::NodeHandle nh;

sensor_msgs::BatteryState battery;

ros::Publisher *battery_pub;

geometry_msgs::PoseStamped pose;

ros::Publisher *pose_pub;

sensor_msgs::Range range_fl;

sensor_msgs::Range range_fr;

sensor_msgs::Range range_rl;

sensor_msgs::Range range_rr;

ros::Publisher *range_pub_fl;

ros::Publisher *range_pub_fr;

ros::Publisher *range_pub_rl;

ros::Publisher *range_pub_rr;

std::vector<float> rosbot_pose;

std::vector<float> rpy;

std::vector<float> ranges;

int publish_counter = 0;

void twistCallback(const geometry_msgs::Twist &twist)

{

rosbot.setSpeed(twist.linear.x, twist.angular.z);

}

void initCmdVelSubscriber()

{

ros::Subscriber<geometry_msgs::Twist> *cmd_sub = new ros::Subscriber<geometry_msgs::Twist>("/cmd_vel", &twistCallback);

nh.subscribe(*cmd_sub);

}

void resetCallback(const std_msgs::Bool &msg)

{

if (msg.data == true)

{

rosbot.reset_odometry();

}

}

void initResetOdomSubscriber()

{

ros::Subscriber<std_msgs::Bool> *odom_reset_sub = new ros::Subscriber<std_msgs::Bool>("/reset_odom", &resetCallback);

nh.subscribe(*odom_reset_sub);

}

void initDistanceSensorsPublisher()

{

range_fl.header.frame_id = "range_fl";

range_fr.header.frame_id = "range_fr";

range_rl.header.frame_id = "range_rl";

range_rr.header.frame_id = "range_rr";

switch (sensor_type)

{

case SENSOR_LASER:

range_fl.field_of_view = 0.26;

range_fl.min_range = 0.03;

range_fl.max_range = 0.90;

range_fr.field_of_view = 0.26;

range_fr.min_range = 0.03;

range_fr.max_range = 0.90;

range_rl.field_of_view = 0.26;

range_rl.min_range = 0.03;

range_rl.max_range = 0.90;

range_rr.field_of_view = 0.26;

range_rr.min_range = 0.03;

range_rr.max_range = 0.90;

break;

case SENSOR_INFRARED:

range_fl.radiation_type = sensor_msgs::Range::INFRARED;

range_fl.field_of_view = 0.26;

range_fl.min_range = 0.05;

range_fl.max_range = 0.299;

range_fr.radiation_type = sensor_msgs::Range::INFRARED;

range_fr.field_of_view = 0.26;

range_fr.min_range = 0.05;

range_fr.max_range = 0.299;

range_rl.radiation_type = sensor_msgs::Range::INFRARED;

range_rl.field_of_view = 0.26;

range_rl.min_range = 0.05;

range_rl.max_range = 0.299;

range_rr.radiation_type = sensor_msgs::Range::INFRARED;

range_rr.field_of_view = 0.26;

range_rr.min_range = 0.05;

range_rr.max_range = 0.299;

break;

case NO_DISTANCE_SENSOR:

// Do your own implementation

break;

}

if (sensor_type != SensorType::NO_DISTANCE_SENSOR)

{

range_pub_fl = new ros::Publisher("/range/fl", &range_fl);

range_pub_fr = new ros::Publisher("/range/fr", &range_fr);

range_pub_rl = new ros::Publisher("/range/rl", &range_rl);

range_pub_rr = new ros::Publisher("/range/rr", &range_rr);

nh.advertise(*range_pub_fl);

nh.advertise(*range_pub_fr);

nh.advertise(*range_pub_rl);

nh.advertise(*range_pub_rr);

}

}

void initBatteryPublisher()

{

battery_pub = new ros::Publisher("/battery", &battery);

nh.advertise(*battery_pub);

}

void initPosePublisher()

{

pose.header.frame_id = "base_link";

pose.pose.orientation = tf::createQuaternionFromYaw(0);

pose_pub = new ros::Publisher("/pose", &pose);

nh.advertise(*pose_pub);

}

void hMain()

{

Serial.printf("init ROSbot\n");

rosbot.initROSbot(sensor_type);

Serial.printf("init with dvice\n");

platform.begin(&RPi);

nh.getHardware()->initWithDevice(&platform.LocalSerial);

nh.initNode();

initBatteryPublisher();

initPosePublisher();

initDistanceSensorsPublisher();

initCmdVelSubscriber();

initResetOdomSubscriber();

while (true)

{

nh.spinOnce();

publish_counter++;

if (publish_counter > 10)

{

// get ROSbot pose

rosbot_pose = rosbot.getPose();

pose.pose.position.x = rosbot_pose[0];

pose.pose.position.y = rosbot_pose[1];

pose.pose.orientation = tf::createQuaternionFromYaw(rosbot_pose[2]);

// publish pose

pose_pub->publish(&pose);

if (sensor_type != SensorType::NO_DISTANCE_SENSOR)

{

// get ranges from distance sensors

ranges = rosbot.getRanges(sensor_type);

range_fl.range = ranges[0];

range_fr.range = ranges[1];

range_rl.range = ranges[2];

range_rr.range = ranges[3];

Serial.printf("Ranges %f %f %f %f\n", ranges[0], ranges[1], ranges[2], ranges[3]);

// publish ranges

range_pub_fl->publish(&range_fl);

range_pub_fr->publish(&range_fr);

range_pub_rl->publish(&range_rl);

range_pub_rr->publish(&range_rr);

}

// get battery voltage

battery.voltage = rosbot.getBatteryLevel();

// publish battery voltage

battery_pub->publish(&battery);

publish_counter = 0;

}

sys.delay(10);

}

}

Copy

Build the project and upload it to your device. Then open action_controller.cpp file in text editor.

Begin with including required header file:

#include <sensor_msgs/Range.h>

Copy

Add variables for measured object distance, average distance and desired distance to obstacle:

float distFL = 0;

float distFR = 0;

float average_dist = 0;

float desired_dist = 0.2;

Copy

Callback functions for incoming sensor messages, their task is only to put values into appropriate variables:

void distFL_callback(const sensor_msgs::Range &range)

{

distFL = range.range;

}

void distFR_callback(const sensor_msgs::Range &range)

{

distFR = range.range;

}

Copy

Then, in switch statement, calculate average distance and set velocity proportional to it only if both sensors found an obstacle, else set zero value for linear velocity:

if (distFL > 0 && distFR > 0)

{

average_dist = (distFL + distFR) / 2;

set_vel.linear.x = (average_dist - desired_dist) / 4;

}

else

{

set_vel.linear.x = 0;

}

Copy

In main function, subscribe to sensor topics:

ros::Subscriber distFL_sub = n.subscribe("/range/fl", 1, distFL_callback);

ros::Subscriber distFR_sub = n.subscribe("/range/fr", 1, distFR_callback);

Copy

Final file should look like this:

#include <ros/ros.h>

#include <std_msgs/Float32MultiArray.h>

#include <std_msgs/Int32MultiArray.h>

#include <geometry_msgs/Twist.h>

#include <std_msgs/String.h>

#include <opencv2/opencv.hpp>

#include <sensor_msgs/Range.h>

#define SMILE 4

#define ARROW_LEFT 3

#define ARROW_UP 5

#define ARROW_DOWN 6

int id = 0;

ros::Publisher action_pub;

geometry_msgs::Twist set_vel;

int camera_center = 320; // left 0, right 640

float max_ang_vel = 0.6;

float min_ang_vel = 0.4;

float ang_vel = 0;

float distFL = 0;

float distFR = 0;

float average_dist = 0;

float desired_dist = 0.2;

void distFL_callback(const sensor_msgs::Range &range)

{

distFL = range.range;

}

void distFR_callback(const sensor_msgs::Range &range)

{

distFR = range.range;

}

void objectCallback(const std_msgs::Float32MultiArrayPtr &object)

{

if (object->data.size() > 0)

{

id = object->data[0];

float objectWidth = object->data[1];

float objectHeight = object->data[2];

float x_pos;

float speed_coefficient = (float)camera_center / max_ang_vel;

// Find corners OpenCV

cv::Mat cvHomography(3, 3, CV_32F);

std::vector<cv::Point2f> inPts, outPts;

switch (id)

{

case ARROW_LEFT:

set_vel.linear.x = 0;

set_vel.angular.z = 1;

break;

case ARROW_UP:

set_vel.linear.x = 1;

set_vel.angular.z = 0;

break;

case ARROW_DOWN:

set_vel.linear.x = -1;

set_vel.angular.z = 0;

break;

case SMILE:

cvHomography.at<float>(0, 0) = object->data[3];

cvHomography.at<float>(1, 0) = object->data[4];

cvHomography.at<float>(2, 0) = object->data[5];

cvHomography.at<float>(0, 1) = object->data[6];

cvHomography.at<float>(1, 1) = object->data[7];

cvHomography.at<float>(2, 1) = object->data[8];

cvHomography.at<float>(0, 2) = object->data[9];

cvHomography.at<float>(1, 2) = object->data[10];

cvHomography.at<float>(2, 2) = object->data[11];

inPts.push_back(cv::Point2f(0, 0));

inPts.push_back(cv::Point2f(objectWidth, 0));

inPts.push_back(cv::Point2f(0, objectHeight));

inPts.push_back(cv::Point2f(objectWidth, objectHeight));

cv::perspectiveTransform(inPts, outPts, cvHomography);

x_pos = (int)(outPts.at(0).x + outPts.at(1).x + outPts.at(2).x +

outPts.at(3).x) /

4;

ang_vel = -(x_pos - camera_center) / speed_coefficient;

if (ang_vel >= -(min_ang_vel / 2) && ang_vel <= (min_ang_vel / 2))

{

set_vel.angular.z = 0;

if (distFL > 0 && distFR > 0)

{

average_dist = (distFL + distFR) / 2;

set_vel.linear.x = (average_dist - desired_dist) / 4;

}

else

{

set_vel.linear.x = 0;

}

}

else if (ang_vel >= max_ang_vel)

{

set_vel.angular.z = max_ang_vel;

}

else if (ang_vel <= -max_ang_vel)

{

set_vel.angular.z = -max_ang_vel;

}

else

{

set_vel.angular.z = ang_vel;

}

break;

default: // other object

set_vel.linear.x = 0;

set_vel.angular.z = 0;

}

action_pub.publish(set_vel);

}

else

{

// No object detected

set_vel.linear.x = 0;

set_vel.angular.z = 0;

action_pub.publish(set_vel);

}

}

int main(int argc, char **argv)

{

std_msgs::String s;

std::string str;

str.clear();

str.append("");

std::to_string(3);

s.data = str;

ros::init(argc, argv, "action_controller");

ros::NodeHandle n("~");

ros::Subscriber sub = n.subscribe("/objects", 1, objectCallback);

ros::Subscriber distL_sub = n.subscribe("/range/fl", 1, distFL_callback);

ros::Subscriber distR_sub = n.subscribe("/range/fr", 1, distFR_callback);

ros::Rate loop_rate(50);

action_pub = n.advertise<geometry_msgs::Twist>("/cmd_vel", 1);

set_vel.linear.x = 0;

set_vel.linear.y = 0;

set_vel.linear.z = 0;

set_vel.angular.x = 0;

set_vel.angular.y = 0;

set_vel.angular.z = 0;

while (ros::ok())

{

ros::spinOnce();

loop_rate.sleep();

}

}

Copy

Now you can build your node and test it.

Task 3 Run your node along with find_object_2d and astra.launch or Gazebo simulator. Place the same object as in Task 2 in front of your robot. Observe how it turns and drives towards the object.

## Summary

After completing this tutorial you should be able to configure your CORE2 device with vision system to recognize objects. You should also be able to determine the position of recognized object relative to camera and create a node that perform specific action related to recognized objects. You should also know how to handle proximity sensors with sensor_msgs/Range message type.

by Łukasz Mitka, Husarion

Do you need any support with completing this tutorial or have any difficulties with software or hardware? Feel free to describe your thoughts on our community forum: https://community.husarion.com/ or to contact with our support: support@husarion.com

← 3. SIMPLE KINEMATICS FOR MOBILE ROBOT5. RUNNING ROS ON MULTIPLE MACHINES →